%N uvtcor

%D Redo the Tsys correction, using a median smoothed Tsys curve

%P nebk

%: calibration

%B

UVTCOR redoes the Tsys correction by constructing a median

smoothed Tsys curve for each specified mosaiced field,

polarization, antenna and frequency.

%A vis

Input visibility file. No default.

%A out

Output visibility file.  No default.

%A freq

Band centre frequencies of interest in MHz. No default.

%A source

Root name of mosaiced field.  E.g. "cena_" with fields like

cena_1, cena_12  etc.  No default.

%A fields

Integer range of fields such as 1,20  No default.

%A device

PGPLOT device.  No default.

%A options

tsysm   means when undoing the Tsys correction, use the "xtsysm"

and "ytsysm" variables instead of "xtsys" and "ytsys".  This

would be appropriate if you have already run UVTCOR once, and

want to redo something.  It will always median smooth the

"xtsys"  and "ytsys" variables

first   means automatically edit out the first cycle of each

visit to each field before determining the median Tsys curve