%N wvcal

%D Estimate antenna gains from WVR measurements.

%P mchw

%: calibration, atmospheric phase

%B

WVCAL estimates atmospheric phase corrections

from WVR measurements. The WVR data are interpolated

to the integration times of the interferometer data

and written into the gains item.

%A vis

Name of input uv-data file. No default.

%A wvr

Name of input WVR data files. WVCAL looks for filenames

wvrN, where N=1,nants. The phase correction is set to zero

if no WVR data file is found for any antenna. No default.

%A pgain

`pgain' gives the antenna phase change with WVR path length in

in units of radians/millimeter. Default = 0. calculates

-2pi/lambda radians/millimeter from the observing frequency.

%A interval

The length of time, in minutes, of a gain solution.

Default uses the interferometer integration interval,

but use a larger value in cases of poor signal to noise, or

if the atmosphere and instrument is fairly stable.

%A timeoff

time offset in seconds for each antenna.

%A out

Name of output uv-data file. The default is to write the gain

corrections into the input visibility file.

%A device

PGPLOT device to plot mmpath versus airmass. Default is no plot.

%A airmass

airmass range (min,max) to use.

default=1,5

%A mmpath

mmpath range (min,max) to use.

default=0,1000.

%A options

Extra processing options. Several can be given, separated by

commas. Minimum match is used. Possible values are:

filter   Filter out data more than 2 sigma from the mean path.

scale    Scale the measured path by the slope for each antenna.

subtract Subtract the average for each antenna.